Satellite Health System Prototype V2 with Machine Learning and Analysis

Over the past two weeks, I developed and tested two versions of a prototype satellite health-analysis and prediction system using Visual Studio Code, Codex, and GitHub.

I built the first version to better understand how Random Forest machine learning works. I then expanded it into a functional and explainable health-monitoring prototype that includes telemetry validation, command context, subsystem-level analysis, model checks, alert persistence, and structured reporting.

System overview

The health system combines canonical satellite housekeeping telemetry with the most recent preceding command issued to the same satellite. It validates and preprocesses the input, constructs 18 model features, and passes them to a Random Forest classifier.

The classifier assigns one of 4 health states:

- Healthy

- Warning

- Degraded

- Critical

The system does not rely only on machine-learning results. It also performs explainable engineering assessments for nine spacecraft health areas, then combines the accepted ML classification with those results to determine the overall health state.

The final report includes:

- Raw ML classification

- Class vote fractions

- Model-applicability decision

- Overall health state

- Individual subsystem assessments

- Stable fault codes and supporting evidence

- Cross-subsystem fault hypotheses

- Alert status and duration

- Data-quality information

- Health trend

- Human-readable observations

- Preliminary health advisories

The report is written as schema-validated JSON for use by downstream digital-twin components.

How the system works

Canonical telemetry + command history
 ↓
Validate telemetry structure and physical values
 ↓
Match each observation to its latest preceding command
 ↓
Create the 18 machine-learning features
 ↓
Run the Random Forest classifier
 ↓
Perform basic model-applicability screening
 ↓
Independently assess nine spacecraft health areas
 ↓
Apply alert confirmation, persistence, and clearing logic
 ↓
Select the most severe trusted health result
 ↓
Generate fault hypotheses, explanations, trends, and advisories
 ↓
Produce a schema-validated digital-twin health report

Machine-learning inputs

The Random Forest uses 18 features: 12 numerical features and 6 categorical features.

Numerical features

- Battery state of charge

- Solar-array voltage

- Solar-array current

- Flight-computer temperature

- Payload temperature

- Reaction-wheel 1 speed

- Reaction-wheel 2 speed

- Reaction-wheel 3 speed

- Downlink rate

- Battery voltage

- Signed battery current

- Seconds since the most recent preceding command

Categorical features

- Spacecraft mode

- Eclipse state

- Solar-array configuration

- Battery-current sign convention

- Most recent command name

- Most recent command status

The shared feature contract is defined in src/health/health_features.py.

Satellite ID and timestamp are used for validation, command alignment, trend analysis, and reporting, but they are deliberately excluded from the model's features. This reduces the risk that the model learns about an artificial relationship between a particular satellite or date and a health label.

Telemetry validation

Operational input must use the canonical housekeeping telemetry contract. Legacy flat telemetry is rejected by the current operational path.

Before prediction, the system checks for:

- Missing required fields

- Incorrect field types

- Structurally invalid records

- Impossible physical values

- Invalid or future timestamps

- Duplicate observations

- Stale measurements

- Inconsistent units or field definitions

Invalid telemetry is rejected before it reaches the model. Some nonfatal conditions, such as stale telemetry, may still be processed, but the input is marked as degraded.

The current model is trained directly on canonical housekeeping field names. It uses signed battery current and all three signed reaction-wheel speed channels without collapsing them into a single maximum-speed feature.

Validation is performed through the common telemetry-validation component, while health-specific preparation is handled by src/health/telemetry_adapter.py and src/health/health_monitor.py.

Command-to-telemetry alignment

For every telemetry observation, the system finds:

The newest command for the same satellite whose timestamp is at or before the telemetry timestamp.

This time-based rule prevents the system from matching telemetry with a command that has not happened yet.

The system records:

- Most recent command name

- Most recent command status

- Command timestamp

- Seconds since the command

This allows the health model and explainable analysis to consider whether an observed change may be related to recent spacecraft activity.

Random Forest classification

The Random Forest contains multiple decision trees. Each tree evaluates combinations of telemetry and command conditions and votes for 1 of the 4 health classes.

The class receiving the greatest number of votes becomes the raw ML prediction.

For example, a 0.85 value for Degraded means approximately 85% of the trees voted for Degraded. It should not yet be interpreted as a validated 85% probability that the spacecraft is operationally degraded.

Because of this, the report labels these values as uncalibrated Random Forest vote fractions.

Model assurance and abstention

The system performs basic model-applicability screening using missing-feature checks, individual numerical ranges, and categorical values observed during training.

The model artifact stores:

- Numerical ranges observed during training

- Categorical values observed during training

- Model configuration and training metadata

The ML result is marked as abstained when:

- A required model value is missing

- A numerical value falls outside the profiled training domain

- A category was not observed during training

- The model artifact lacks the required training profile

When the model abstains, its raw prediction remains available for diagnostic review, but it is excluded from the overall-health calculation. The explainable subsystem assessments can still determine health.

This keeps the system from treating an unfamiliar input as a trusted ML prediction.

Passing this screening still does not prove that the full observation closely matches the overall training data distribution.

Explainable subsystem analysis

The prototype independently evaluates nine spacecraft health areas:

- Power

- Thermal

- Payload

- Attitude Determination and Control System

- Communications

- Command and Data Handling

- Propulsion

- Timing

- Command Execution

Command Execution checks command-queue depth and recent command completion status. It represents an operational health function rather than a separate hardware subsystem, so it is different from C&DH resource monitoring.

Each  health subsystem assessment can include:

- Health status

- Confidence

- Fault codes

- Observed values

- Prototype thresholds

- Supporting evidence

- Rates of change

- Operational context

- Data completeness

- Known limitations

Missing subsystem telemetry produces an Unknown result rather than incorrectly producing Healthy.

An Unknown result is kept with the data-completeness information, but it does not override a known Healthy, Warning, Degraded, or Critical result. If the ML result is rejected and none of the health areas has enough information for a known status, the overall health state may be Unknown.

The engineering thresholds are currently prototype assumptions. They must eventually be replaced or approved using spacecraft-specific operating limits.

Context-aware analysis

The system considers operational context before identifying some conditions as faults.

For example, low solar-array current may be normal when:

- The spacecraft is in eclipse

- The solar arrays are stowed

Low solar current becomes more concerning when the spacecraft is in sunlight, and the arrays are deployed.

Similarly, a low downlink rate may be expected outside a communications pass but concerning during an active pass.

This context helps reduce false alerts.

Alert persistence

Confirmation and clearing logic are applied separately to each explainable health-area assessment. The overall-health calculation then combines those persistence-adjusted statuses with the accepted raw ML classification. The ML result and final combined result do not currently use their own persistence states.

Overall predictions are saved in the configured JSON history file for health-trend comparisons. Previous telemetry and subsystem alert states are currently held in memory while a report batch runs. This means confirmation counts and alert durations apply only to that batch and are not restored the next time the monitor runs.

The prototype includes confirmation and clearing logic to prevent brief sensor spikes from causing unstable alerts.

- Warning and Degraded conditions require two consecutive observations for confirmation.

- Critical conditions are confirmed immediately.

- An active alert requires two consecutive healthy observations before it clears.

- Missing data does not automatically clear an existing alert.

The report also records:

- Whether an alert is observed, confirmed, clearing, or clear

- Number of consecutive observations

- Time the alert became active

- Alert duration

Overall health determination

The final overall health state uses a conservative maximum-severity method.

The system compares:

- The ML classification, if accepted by model assurance

- Power status

- Thermal status

- Payload status

- ADCS status

- Communications status

- C&DH status

- Propulsion status

- Timing status

- Command Execution status

The most severe trusted result becomes the overall health state.

The output uses schema version 1.1.0 and is validated against docs/requirements/health/health_predictions.schema.json.

For example:

ML classification: Healthy
	Power: Warning
	Thermal: Healthy
	ADCS: Critical

	Overall health: Critical
	Primary contributor: ADCS

This prevents a serious and explainable subsystem condition from being hidden by a less severe ML classification.

Cross-subsystem fault isolation

The system correlates related subsystem symptoms and generates candidate fault hypotheses.

Examples include:

- Power degradation contributing to communications degradation

- Electrical overcurrent occurring with thermal stress

- Reaction-wheel stress contributing to elevated power demand

- Communications degradation affecting command execution

These outputs are labeled as possible fault hypotheses rather than confirmed root causes. Their purpose is to help guide the operator's investigation.

Current maturity and limitations

The prototype shows that the software architecture and end-to-end workflow work as intended. However, it is not a flight-qualified health-management system.

Current limitations include:

- The model is trained with synthetic prototype data.

- Engineering thresholds are prototype assumptions.

- Random Forest vote fractions are not calibrated as operational probabilities.

- Independent operational telemetry has not yet been used for validation.

- Hardware-in-the-loop validation remains for future work.

- Recommendations are preliminary health advisories, not final operator courses of action.

- Model-applicability screening is feature-wise and does not establish multidimensional similarity.

- Subsystem alert-persistence state does not currently survive separate monitor executions.

The next step is to validate and calibrate the system using representative spacecraft telemetry, approved subsystem limits, injected fault scenarios, software simulations, and eventually hardware-in-the-loop testing.